#include <includes.h>

#include "func.h"

void InitBoard(void);

/*--------------- AAPLICATION STACKS ---------*/

static  OS_STK  AppTask0stk[256];       /* Create the required number of stacks need for every child task*/

static  OS_STK  AppTask1stk[256];

/*--------------- CHILDERN TRASKS --------------*/

static  void  AppTask0(void *p_arg);

static  void  AppTask1(void *p_arg);

OS_EVENT *TxMbox;

int  main (void)

{

InitBoard();

OSInit();                                 /* Initialize "uC/OS-II, The Real-Time Kernel"              */

/*----------write your application here------------------*/

TxMbox  = OSMboxCreate((void *)0);      /* Create Mailbox   */

OSTaskCreate(AppTask0,0,&AppTask0stk[256-1],4);

OSTaskCreate(AppTask1,0,&AppTask1stk[256-1],5);

/*---------------------------------------------------------*/

OSStart();                                                  /* Start multitasking (i.e. give control to uC/OS-II)       */

}

static  void  AppTask0 (void *p_arg)

{

unsigned char msg;

unsigned int i;

p_arg = p_arg;                                     /*Just to avoid compiler Warning             */

while(1)

{

printf("TASK0: press any key on PC Keyboard\n\r");

OSTimeDlyHMSM(0,0,0,100);

msg = UART_GetChar();

OSMboxPost(TxMbox,&msg);

OSTimeDlyHMSM(0,0,1,0);

}

}

static void AppTask1(void *p_arg)

{

unsigned char err, *msg;

p_arg = p_arg;                                           /* Just to avoid compiler Warning          */

printf("TASK1: Waiting for message from Mailbox\n\r");

while(1)

{

/* User Code Here */

msg = (unsigned char*)OSMboxPend(TxMbox,0,&err);

printf("TASK1: Message received....\n\r\n\r");

LCD_data(*msg);

OSTimeDlyHMSM(0,0,1,0);

}

}

void InitBoard(void)

{

BSP_IntDisAll();                          /* Disable all interrupts until we are ready to accept them */

BSP_Init();

InitLCD();                              /* Initialize LCD   */

kbdInit();                              /* Initialize Keyboard  */

ADCInit();                              /* Initialize ADC   */

LEDInit();                              /* Initialize LED   */

UartInit(9600);                         /* Initialise the UART*/

}