MOD N-counter code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity N_bit_counter is

generic (

N : integer := 4

);

Port (

clk   : in  STD_LOGIC;

reset : in  STD_LOGIC;

count : out STD_LOGIC_VECTOR(N-1 downto 0)

);

end N_bit_counter;

architecture Behavioral of N_bit_counter is

signal counter : unsigned(N-1 downto 0) := (others => '0');

-- Clock divider counter

signal clk_div : unsigned(26 downto 0) := (others => '0');

-- Divided 1 Hz clock

signal clk_1Hz : STD_LOGIC := '0';

begin

-- Process 1: Divide 100 MHz clock to 1 Hz

process(clk, reset)

begin

if reset = '1' then

clk_div <= (others => '0');

clk_1Hz <= '0';

elsif rising_edge(clk) then

if clk_div = 49_999_999 then

clk_div <= (others => '0');

clk_1Hz <= not clk_1Hz;

else

clk_div <= clk_div + 1;

end if;

end if;

end process;

-- Process 2: Your original counter

process(clk_1Hz, reset)

begin

if reset = '1' then

counter <= (others => '0');

elsif rising_edge(clk_1Hz) then

if counter = 9 then

counter <= (others => '0');

else

counter <= counter + 1;

end if;

end if;

end process

count <= std_logic_vector(counter);

end Behavioral;

Test Bench code:-

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity N_bit_counter_tb is

end N_bit_counter_tb;

architecture Behavioral of N_bit_counter_tb is

-- Component Declaration

component N_bit_counter

generic (

N : integer := 4

);

Port (

clk   : in  STD_LOGIC;

reset : in  STD_LOGIC;

count : out STD_LOGIC_VECTOR(N-1 downto 0)

);

end component;

-- Testbench signals

signal clk   : STD_LOGIC := '0';

signal reset : STD_LOGIC := '0';

signal count : STD_LOGIC_VECTOR(3 downto 0);

begin

-- Instantiate DUT

uut: N_bit_counter

generic map (

N => 4

)

port map (

clk   => clk,

reset => reset,

count => count

);

-- 100 MHz Clock

-- Period = 10 ns

clk_process : process

begin

while true loop

clk <= '0';

wait for 5 ns;

clk <= '1';

wait for 5 ns;

end loop;

end process;

-- Reset and simulation

stimulus_process : process

begin

-- Apply reset

reset <= '1';

wait for 100 ns;

-- Release reset

reset <= '0';

-- Wait for counter operation

wait for 2 sec;

-- Apply reset again

reset <= '1';

wait for 100 ns;

reset <= '0';

wait for 2 sec;

-- End simulation

wait;

end process;

end Behavioral;

Simulation Waveform:

UCF code:-

set_property PACKAGE_PIN L1 [get_ports {count[3]}]

set_property PACKAGE_PIN P1 [get_ports {count[2]}]

set_property PACKAGE_PIN N3 [get_ports {count[1]}]

set_property PACKAGE_PIN P3 [get_ports {count[0]}]

set_property PACKAGE_PIN W5 [get_ports clk]

set_property PACKAGE_PIN R2 [get_ports reset]

set_property IOSTANDARD LVCMOS33 [get_ports {count[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {count[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {count[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {count[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports reset]