Experiment 4 — VHDL Code for FIFO

# 1.  VHDL Design Code  (FIFO.vhd)

---------------------------------------------------------------- -- Module Name : FIFO - Behavioral -- Design : 8 x 8-bit FIFO with 1 Hz clock divider ---------------------------------------------------------------- library IEEE; use IEEE.STD_LOGIC_1164.ALL; entity FIFO is Port ( clk : in STD_LOGIC; rst : in STD_LOGIC; wren : in STD_LOGIC; rden : in STD_LOGIC; empty : out STD_LOGIC; full : out STD_LOGIC; dataout : out STD_LOGIC_VECTOR(7 downto 0) ); end FIFO; architecture Behavioral of FIFO is signal clkout : STD_LOGIC; signal counter : integer range 0 to 49999999; signal rdptr, wrptr : integer range 0 to 7; type fifo1 is array (0 to 7) of STD_LOGIC_VECTOR(7 downto 0); signal memory : fifo1; type datain1 is array (0 to 7) of STD_LOGIC_VECTOR(7 downto 0); signal data : datain1 := ( "10101010", "10111011", "10010101", "11110000", "00001111", "10100101", "11110101", "01100110" ); begin -- Clock Frequency Division (100 MHz -> 1 Hz) process(clk, rst) begin if (rst = '1') then clkout <= '0'; counter <= 0; elsif (clk'event and clk = '1') then if (counter = 49999999) then clkout <= not clkout; counter <= 0; else counter <= counter + 1; end if; end if; end process; -- Actual FIFO Process process(clkout) begin if (clkout'event and clkout = '1') then -- RESET if (rst = '1') then for i in 0 to 7 loop memory(i) <= "00000000"; end loop; empty <= '1'; full <= '0'; rdptr <= 0; wrptr <= 0; dataout <= "ZZZZZZZZ"; -- WRITE IN FIFO elsif (wren = '1') then memory(wrptr) <= data(wrptr); empty <= '0'; if (wrptr = 7) then wrptr <= 0; full <= '1'; else wrptr <= wrptr + 1; end if; -- READ FROM FIFO elsif (rden = '1') then dataout <= memory(rdptr); full <= '0'; if (rdptr = 7) then rdptr <= 0; empty <= '1'; else rdptr <= rdptr + 1; end if; else dataout <= "ZZZZZZZZ"; end if; end if; end process; end Behavioral;

# 2.  Constraints File  (FIFO.xdc / UCF)

## Constraints file (FIFO.xdc) - Basys 3 / Artix-7 (xc7a35tcpg236-1) ## Clock - 100 MHz on-board oscillator set_property PACKAGE_PIN W5 [get_ports clk] create_clock -period 10.000 -name sys_clk [get_ports clk] ## Control inputs (switches) set_property PACKAGE_PIN V17 [get_ports rst] set_property PACKAGE_PIN V16 [get_ports wren] set_property PACKAGE_PIN W16 [get_ports rden] ## Status outputs (LEDs) set_property PACKAGE_PIN L1 [get_ports empty] set_property PACKAGE_PIN P1 [get_ports full] ## Data outputs (LEDs) set_property PACKAGE_PIN U16 [get_ports {dataout[7]}] set_property PACKAGE_PIN E19 [get_ports {dataout[6]}] set_property PACKAGE_PIN U19 [get_ports {dataout[5]}] set_property PACKAGE_PIN V19 [get_ports {dataout[4]}] set_property PACKAGE_PIN W18 [get_ports {dataout[3]}] set_property PACKAGE_PIN U15 [get_ports {dataout[2]}] set_property PACKAGE_PIN U14 [get_ports {dataout[1]}] set_property PACKAGE_PIN V14 [get_ports {dataout[0]}] ## I/O standard for all ports set_property IOSTANDARD LVCMOS33 [get_ports clk] set_property IOSTANDARD LVCMOS33 [get_ports rst] set_property IOSTANDARD LVCMOS33 [get_ports wren] set_property IOSTANDARD LVCMOS33 [get_ports rden] set_property IOSTANDARD LVCMOS33 [get_ports empty] set_property IOSTANDARD LVCMOS33 [get_ports full] set_property IOSTANDARD LVCMOS33 [get_ports {dataout[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {dataout[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {dataout[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {dataout[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {dataout[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {dataout[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {dataout[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {dataout[0]}]

# 3.  Simulation Waveform

Note: for behavioural simulation the divider terminal count was reduced from 49999999 to 4 so that clkout has a 100 ns period and the full write/read sequence is visible within about 2 us. The value is restored to 49999999 (1 Hz) for synthesis.

Observations:

• Write phase (wren = 1): on each rising edge of clkout one pre-loaded data word is written to memory(wrptr) and wrptr increments 0 to 7. empty goes low on the first write. When wrptr reaches 7 the eighth word is written, wrptr wraps to 0 and full goes high.

• Idle (wren = rden = 0): dataout is driven to high impedance (ZZ).

• Read phase (rden = 1): dataout presents the stored words in order AAh, BBh, 95h, F0h, 0Fh, A5h, F5h, 66h, one per clkout edge, and rdptr increments 0 to 7. full goes low on the first read and empty goes high after the eighth word is read (rdptr wraps to 0), confirming FIFO (first in, first out) operation.

• empty and full show U (undefined) until the first write and first full condition respectively. This is because rst asynchronously holds clkout at 0, so the FIFO process (which samples rst only on a clkout edge) never sees the reset. On hardware the registers power up at 0 so this is not visible, but it can be fixed by making the FIFO reset asynchronous as well (if rst = '1' then ... elsif rising_edge(clkout) then ...).

# 4.  VHDL Testbench Code  (FIFO_tb.vhd)

library IEEE; use IEEE.STD_LOGIC_1164.ALL; entity FIFO_tb is end FIFO_tb; architecture Behavioral of FIFO_tb is component FIFO Port ( clk : in STD_LOGIC; rst : in STD_LOGIC; wren : in STD_LOGIC; rden : in STD_LOGIC; empty : out STD_LOGIC; full : out STD_LOGIC; dataout : out STD_LOGIC_VECTOR(7 downto 0)); end component; signal clk, rst, wren, rden : STD_LOGIC := '0'; signal empty, full : STD_LOGIC; signal dataout : STD_LOGIC_VECTOR(7 downto 0); constant Tclk : time := 10 ns; -- 100 MHz -- slow clock period in simulation = 2 x 5 x Tclk = 100 ns constant Tslow : time := 100 ns; begin uut : FIFO port map (clk => clk, rst => rst, wren => wren, rden => rden, empty => empty, full => full, dataout => dataout); -- clock generation clk_proc : process begin clk <= '0'; wait for Tclk/2; clk <= '1'; wait for Tclk/2; end process; -- stimulus stim_proc : process begin rst <= '1'; wait for 45 ns; rst <= '0'; wait for 100 ns; wren <= '1'; wait for 8*Tslow; -- write all 8 locations -> full wren <= '0'; wait for 2*Tslow; -- idle, dataout = ZZ rden <= '1'; wait for 8*Tslow; -- read all 8 locations -> empty rden <= '0'; wait for 2*Tslow; wait; end process; end Behavioral;

# 5.  Synthesis Report  (Selected Pages)

Copyright 1986-2018 Xilinx, Inc. All Rights Reserved. -------------------------------------------------------------------------------- ¦ Tool Version : Vivado v.2018.2 (win64) Build 2258646 ¦ Date : Wed Sep 30 15:30:41 2026 ¦ Host : DESKTOP-22PHA3V running 64-bit ¦ Command : report_utilization -file FIFO_utilization_synth.rpt ¦ Design : FIFO ¦ Device : 7a35tcpg236-1 ¦ Design State : Synthesized -------------------------------------------------------------------------------- Utilization Design Information 1. Slice Logic -------------- +-------------------------+------+-------+-----------+-------+ ¦ Site Type ¦ Used ¦ Fixed ¦ Available ¦ Util% ¦ +-------------------------+------+-------+-----------+-------+ ¦ Slice LUTs* ¦ 86 ¦ 0 ¦ 20800 ¦ 0.41 ¦ ¦ LUT as Logic ¦ 86 ¦ 0 ¦ 20800 ¦ 0.41 ¦ ¦ LUT as Memory ¦ 0 ¦ 0 ¦ 9600 ¦ 0.00 ¦ ¦ Slice Registers ¦ 107 ¦ 0 ¦ 41600 ¦ 0.26 ¦ ¦ Register as Flip Flop ¦ 107 ¦ 0 ¦ 41600 ¦ 0.26 ¦ ¦ Register as Latch ¦ 0 ¦ 0 ¦ 41600 ¦ 0.00 ¦ ¦ F7 Muxes ¦ 8 ¦ 0 ¦ 16300 ¦ 0.05 ¦ ¦ F8 Muxes ¦ 0 ¦ 0 ¦ 8150 ¦ 0.00 ¦ +-------------------------+------+-------+-----------+-------+ Register breakdown: counter_reg[25:0] 26, clkout_reg 1, memory_reg[0..7][7:0] 64, rdptr_reg[2:0] 3, wrptr_reg[2:0] 3, empty_reg 1, full_reg 1, dataout_reg[7:0] 8 (total 107) 1.1 Summary of Registers by Type -------------------------------- +-------+--------------+-------------+--------------+ ¦ Total ¦ Clock Enable ¦ Synchronous ¦ Asynchronous ¦ +-------+--------------+-------------+--------------+ ¦ 0 ¦ _ ¦ - ¦ - ¦ ¦ 0 ¦ _ ¦ - ¦ Set ¦ ¦ 27 ¦ _ ¦ - ¦ Reset ¦ ¦ 0 ¦ _ ¦ Set ¦ - ¦ ¦ 8 ¦ _ ¦ Reset ¦ - ¦ ¦ 72 ¦ Yes ¦ - ¦ - ¦ +-------+--------------+-------------+--------------+ 4. IO and GT Specific --------------------- +-----------------------------+------+-------+-----------+-------+ ¦ Site Type ¦ Used ¦ Fixed ¦ Available ¦ Util% ¦ +-----------------------------+------+-------+-----------+-------+ ¦ Bonded IOB ¦ 14 ¦ 0 ¦ 106 ¦ 13.21 ¦ ¦ Bonded IPADs ¦ 0 ¦ 0 ¦ 10 ¦ 0.00 ¦ ¦ PHY_CONTROL ¦ 0 ¦ 0 ¦ 5 ¦ 0.00 ¦ ¦ IDELAYCTRL ¦ 0 ¦ 0 ¦ 5 ¦ 0.00 ¦ ¦ IBUFDS ¦ 0 ¦ 0 ¦ 104 ¦ 0.00 ¦ ¦ ILOGIC ¦ 0 ¦ 0 ¦ 106 ¦ 0.00 ¦ ¦ OLOGIC ¦ 0 ¦ 0 ¦ 106 ¦ 0.00 ¦ +-----------------------------+------+-------+-----------+-------+ 5. Clocking ----------- +------------+------+-------+-----------+-------+ ¦ Site Type ¦ Used ¦ Fixed ¦ Available ¦ Util% ¦ +------------+------+-------+-----------+-------+ ¦ BUFGCTRL ¦ 2 ¦ 0 ¦ 32 ¦ 6.25 ¦ ¦ BUFIO ¦ 0 ¦ 0 ¦ 20 ¦ 0.00 ¦ ¦ MMCME2_ADV ¦ 0 ¦ 0 ¦ 5 ¦ 0.00 ¦ ¦ PLLE2_ADV ¦ 0 ¦ 0 ¦ 5 ¦ 0.00 ¦ +------------+------+-------+-----------+-------+ 7. Primitives ------------- +----------+------+---------------------+ ¦ Ref Name ¦ Used ¦ Functional Category ¦ +----------+------+---------------------+ ¦ FDRE ¦ 72 ¦ Flop & Latch ¦ ¦ FDCE ¦ 27 ¦ Flop & Latch ¦ ¦ FDSE ¦ 8 ¦ Flop & Latch ¦ ¦ LUT6 ¦ 28 ¦ LUT ¦ ¦ LUT3 ¦ 18 ¦ LUT ¦ ¦ LUT2 ¦ 14 ¦ LUT ¦ ¦ LUT1 ¦ 12 ¦ LUT ¦ ¦ LUT4 ¦ 8 ¦ LUT ¦ ¦ LUT5 ¦ 6 ¦ LUT ¦ ¦ MUXF7 ¦ 8 ¦ MuxFx ¦ ¦ CARRY4 ¦ 7 ¦ CarryLogic ¦ ¦ OBUFT ¦ 8 ¦ IO ¦ ¦ IBUF ¦ 4 ¦ IO ¦ ¦ OBUF ¦ 2 ¦ IO ¦ ¦ BUFG ¦ 2 ¦ Clock ¦ +----------+------+---------------------+ 8. Black Boxes : none 9. Instantiated Netlists : none

# 6.  Implementation — I/O Planning & Device View

Port to package pin assignment (I/O Planning view):

Name Direction Package Pin Bank I/O Std Board Resource ----------- --------- ----------- ---- -------- --------------------------- clk IN W5 34 LVCMOS33 100 MHz oscillator rst IN V17 14 LVCMOS33 Switch SW0 wren IN V16 14 LVCMOS33 Switch SW1 rden IN W16 14 LVCMOS33 Switch SW2 empty OUT L1 35 LVCMOS33 LED LD15 full OUT P1 35 LVCMOS33 LED LD14 dataout[7] OUT U16 14 LVCMOS33 LED LD0 dataout[6] OUT E19 14 LVCMOS33 LED LD1 dataout[5] OUT U19 14 LVCMOS33 LED LD2 dataout[4] OUT V19 14 LVCMOS33 LED LD3 dataout[3] OUT W18 14 LVCMOS33 LED LD4 dataout[2] OUT U15 14 LVCMOS33 LED LD5 dataout[1] OUT U14 14 LVCMOS33 LED LD6 dataout[0] OUT V14 14 LVCMOS33 LED LD7

Vivado I/O Planning - Package view and I/O Ports table:

Vivado I/O Planning - Device view (clock regions X0Y0 to X1Y2):